MATERIAL SAFETY DATA SHEET

CONTINENTAL HYDRODYNE SYSTEMS, INC.                              				PAGE: 1 of  2

1025 Mary Laidley Drive

Covington, KY 41017							Emergency Phone: 1-859-356-2008

SECTION I-PRODUCT IDENTIFICATION

PRODUCT NAME: ARSENIC TEST KIT      CAT# 40005 & 40005RF

SYNONYMS: N/A

MOLECULAR WEIGHT: N/A

COMPONENTS IN BOTTLES:

REAGENT A:

Hydrochloric Acid, 37%        		CAS# [7647-01-0]           	HAZARD: CORROSIVE

REAGENT B:

Potassium Iodide solution               		CAS# [7681-11-0]    		HAZARD: N/A

REAGENT C:

Stannous Chloride solution  			CAS# [N/A]		           HAZARD: CORROSIVE

REAGENT D:

Lead Acetate Solution			CAS# [6080-56-4]		HAZARD: TOXIC

INDICATOR SOLUTION			CAS# N/A			HAZARD: FLAMMABLE LIQUID

CAPSULES: ZINC				CAS#[7440-66-6]		HAZARD: N/A

SECTION II-PHYSICAL DATA

BOILING POINT: N/A	     	  	SP. GRAVITY: N/A

MELTING POINT: N/A             		SOLUBILITY (WATER): yes

APPEARANCE: Reagent A: clear, colorless liquid, w/ pungent odor

Reagent B: clear, colorless to slightly yellow liquid

Reagent C: clear, colorless liquid w/ pungent odor,   Reagent D: White, cloudy liquid

Indicator Solution: clear, yellow liquid w/ pungent odor	   Capsules: Granular/metallic

SECTION III-FIRE AND EXPLOSION HAZARD

FLASH POINT: N/A        		LEL: Not Known    		UEL: Not Known

EXTINGUISHING MEDIA: Dry Chemical, CO2, water

SPECIAL FIRE FIGHTING PROCEDURES: Wear full protective equipment including self-contained breathing apparatus (eye, body, respiratory)

UNUSUAL FIRE HAZARDS: This product contains Pyridine which burns readily upon contact with heat sources.

SECTION IV-REACTIVITY DATA

INSTABILITY: Stable at ambient temperature, avoid contact with heat, flame or strong acids or bases.  Store in a cool, dark, dry area away from direct sunlight and possible heat sources.  This product contains Pyridine which burns readily in presence of heat, sparks or flame.  Reagent B will turn yellow upon contact with air.                   HAZARDOUS POLYMERIZATION: Unknown		DECOMPOSITION PRODUCTS: Unknown.

INCOMPATIBLES: Reagent A & C: Bases       	Indicator Solution: Acids

CONTINENTAL HYDRODYNE SYSTEMS						Page 2 of 2.

MATERIAL SAFETY DATA SHEET                 						REVISED: 11/00

PRODUCT NAME: ARSENIC TEST KIT    CAT# 40005 & 40005RF

HAZARD INDEX:

Reagent A:      HEALTH = 3			FLAMMABILITY = 0            	REACTIVITY = 2

Reagent B:      N/A				Reagent C:      N/A

Reagent D:	HEALTH = 2		            FLAMMABILITY = 0  		REACTIVITY = 0

Indicator:	HEALTH = 2			FLAMMABILITY = 3		REACTIVITY = 1

Capsule:	HEALTH = 0			FLAMMABILITY = 1		REACTIVITY = 2

SECTION V-SPILL, LEAK AND DISPOSAL PROCEDURES

SPILL & DISPOSAL: Contain, collect and dispose all of the waste in accordance with applicable local, state and federal laws.

SECTION VI-SPECIAL PROTECTIVE EQUIPMENT

Use in adequately ventilated area (Fume Hood is recommended),  Utilize protective equipment; chemical splash goggles, rubber gloves and apron.

SECTION VII-HEALTH HAZARD DATA

TLV: Not known    MUTAGENICITY: Not known   LD50: Not known

EFFECTS OF OVEREXPOSURE:  Prolonged and repeated contact with these reagents may be harmful and could cause severe burns and/or irritation with contact on the skin, eyes, and mucous membranes.  Always avoid contact with skin, eyes and mucous membranes.  Do not breathe vapors.  Reagent A & C contains concentrated Hydrochloric Acid and are extremely corrosive to the skin, eyes and mucous membranes.  Handle all reagents with care.

FIRST AID: In case of contact with skin, rinse the affected area with large amounts of water for at least 15 minutes.  Wash contaminated clothing before reuse.  If irritation occurs, seek medical advice.  In case of contact with eyes, flush with water for 15 minutes and seek medical attention.  If inhalation of vapors cause loss of consciousness, move the victim to open air.  Administer artificial respiration or oxygen as necessary.  Seek medical attention.

SECTION VIII-SPECIAL PRECAUTIONS

ALWAYS USE PERSONAL PROTECTIVE EQUIPMENT AND FOLLOW SAFE LABORATORY PRACTICES DURING HANDLING AND STORAGE OF THIS CHEMICAL.  HANDLE ONLY IN WELL VENTILATED AREAS.

THE DATA PROVIDED IS CORRECT TO THE BEST OF OUR KNOWLEDGE.  WE SHALL NOT BE HELD LIABLE FOR ANY DAMAGES RESULTING FROM HANDLING OR CONTACT WITH THIS PRODUCT.