MATERIAL SAFETY DATA SHEET

CONTINENTAL HYDRODYNE SYSTEMS, INC.                              		PAGE: 1 of 2

1025 Mary Laidley Drive

Covington, KY 41017							Emergency Phone: 1-800-255-3924

SECTION I-PRODUCT IDENTIFICATION

PRODUCT NAME: Acetone

SYNONYMS: Dimethyl Ketone, 2-propanone, Dimethal Ketal

MOLECULAR WEIGHT: CH3COCH3

CAS# [67-64-1]

Formula Weight: 58.08 g/mol

HAZARDOUS INFORMATION: This material is a flammable liquid        D.O.T. UN 1090, Class 3, PG II

SECTION II-PHYSICAL DATA

BOILING POINT: 56-57oC / 133-134oF			SP. GRAVITY: 0.792 (H2O = 1)

FREEZING POINT: -97oC / -142oF				VAPOR PRESSURE: 185 mmHg @ 20oC

SOLUBILITY (WATER): miscible w/ water		VOLATILES: yes, 100%

ODOR: Mint							APPEARANCE: clear, colorless liquid

FLASH POINT: -15oC / 4oF (open cup)			VAPOR DENSITY: 2.0 (air = 1)

SECTION III-FIRE AND EXPLOSION HAZARD

LEL: Not Known    UEL: Not Known

EXTINGUISHING MEDIA: Dry Chemical, CO2, water

SPECIAL FIRE FIGHTING PROCEDURES: Wear full protective equipment including self-contained breathing apparatus (eye, body, respiratory)

UNUSUAL FIRE HAZARDS: Evaporates readily.  Highly flammable.  Avoid inhalation of vapor.

STORAGE: Store in a cool, dry place that is well ventilated and away from excessive heat or sources of ignition..

SECTION IV-REACTIVITY DATA

INSTABILITY: Stable at normal, ambient temperatures.  Keep away from excessive heat & sources of ignition.  Vapor may travel by air currents and be ignited by pilot lights, flames, smoking, sparks, heaters, static charges or other sources of ignition from locations distant from handling point.  Vapors may settle in low confined areas and may flash back explosively upon contact with an ignition source.

HAZARDOUS POLYMERIZATION: N/A

DECOMPOSITION PRODUCTS: Caron Monoxide & Carbon Dioxide

INCOMPATIBLES: Alkali metals, acetone+alkali and similar combinations.

HAZARD INDEX:      HEALTH = 1	FLAMMABILITY = 3            REACTIVITY = 2

SECTION V-SPILL, LEAK AND DISPOSAL PROCEDURES

SPILL & DISPOSAL: Ventilate area.  Do not turn on any ignition source until area is determined to be free from fire or explosion hazard.  Collect spilled material with absorbent and dispose of all of the waste in accordance with applicable local, state and federal laws.

CONTINENTAL HYDRODYNE SYSTEMS					Page 2 of 2.

MATERIAL SAFETY DATA SHEET                 					REVISED: 5/2001

PRODUCT NAME: Acetone

SECTION VI-SPECIAL PROTECTIVE EQUIPMENT

Use in adequately ventilated area,  Utilize protective equipment; chemical splash goggles, rubber gloves and apron. Mechanical hood.

SECTION VII-HEALTH HAZARD DATA

TLV: Not known    		MUTAGENICITY: Not known   		LD50: Not known

ACUTE EFFECTS: Skin and eye irritant.  Vapors are irritating and exposure to eyes, nose, throat may cause stinging or itching sensation, tear production, nausea and possibly vomiting.

EFFECTS OF OVEREXPOSURE:  Prolonged and repeated contact with this chemical may be harmful and could cause irritation with contact on the skin, eyes, and mucous membranes.  Always avoid contact with skin, eyes and mucous membranes.  Do not breathe vapors.

FIRST AID: In case of contact with skin, rinse the affected area with large amounts of water for at least 15 minutes.  Wash contaminated clothing before reuse.  If irritation occurs, seek medical advice.  In case of contact with eyes, flush with water for 15 minutes and seek medical attention.  If inhalation of vapors cause loss of consciousness, move the victim to open air.  Administer artificial respiration or oxygen as necessary.  Seek medical attention.  If swallowed, give water, but DO NOT induce vomiting.

SECTION VIII-SPECIAL PRECAUTIONS

ALWAYS USE PERSONAL PROTECTIVE EQUIPMENT AND FOLLOW SAFE LABORATORY PRACTICES DURING HANDLING AND STORAGE OF THIS CHEMICAL.  HANDLE ONLY IN WELL VENTILATED AREAS.

THE DATA PROVIDED IS CORRECT TO THE BEST OF OUR KNOWLEDGE.  WE SHALL NOT BE HELD LIABLE FOR ANY DAMAGES RESULTING FROM HANDLING OR CONTACT WITH THIS PRODUCT.