SECTION I-PRODUCT IDENTIFICATION

PRODUCT NAME: Chloroform

SYNONYMS: Trichloromethane

MOLECULAR WEIGHT: CHCl3

CAS# [67-66-3]

Formula Weight: 119.39

HAZARDOUS INFORMATION: This material is toxic!         D.O.T. class 6.1, PG III

SECTION II-PHYSICAL DATA

BOILING POINT: 61-62oC					SP. GRAVITY: 1.484

FREEZING POINT: -63.5oC					VAPOR PRESSURE: 100mm @ 10.4oC

SOLUBILITY (WATER): not miscible w/ water		VOLATILES: yes, 100%

SECTION III-FIRE AND EXPLOSION HAZARD

FLASH POINT: N/A        LEL: Not Known    UEL: Not Known

EXTINGUISHING MEDIA: Dry Chemical, CO2, water

SPECIAL FIRE FIGHTING PROCEDURES: Wear full protective equipment including self-contained breathing apparatus (eye, body, respiratory)

UNUSUAL FIRE HAZARDS: Evaporates readily.  Not flammable.  Avoid inhalation of vapor.

STORAGE: Store in a cool, dry place that is well ventilated.

SECTION IV-REACTIVITY DATA

INSTABILITY: Stable.  Reacts violently with acetone+alkali and similar combinations.  Incompatible with alkali metals.

HAZARDOUS POLYMERIZATION: N/A

DECOMPOSITION PRODUCTS: N/A

INCOMPATIBLES: Alkali metals, acetone+alkali and similar combinations.

HAZARD INDEX:      HEALTH = 3	FLAMMABILITY = 0            REACTIVITY = 1

SECTION V-SPILL, LEAK AND DISPOSAL PROCEDURES

SPILL & DISPOSAL: Ventilate area.  Collect spilled material with absorbent and dispose all of the waste in accordance with applicable local, state and federal laws.

SECTION VI-SPECIAL PROTECTIVE EQUIPMENT

Use in adequately ventilated area,  Utilize protective equipment; chemical splash goggles, rubber gloves and apron. Mechanical hood.

SECTION VII-HEALTH HAZARD DATA

TLV: Not known    		MUTAGENICITY: Not known   		LD50: Not known

ACUTE EFFECTS: Skin and eye irritant.  This material is toxic by inhalation or oral.  Chloroform is a suspect carcinogen.  This material is known to the state of California to cause cancer.

EFFECTS OF OVEREXPOSURE:  Prolonged and repeated contact with this chemical may be harmful and could cause irritation with contact on the skin, eyes, and mucous membranes.  Always avoid contact with skin, eyes and mucous membranes.  Do not breathe vapors.  This material is a suspect Carcinogen.

FIRST AID: In case of contact with skin, rinse the affected area with large amounts of water for at least 15 minutes.  Wash contaminated clothing before reuse.  If irritation occurs, seek medical advice.  In case of contact with eyes, flush with water for 15 minutes and seek medical attention.  If inhalation of dusts/vapors cause loss of consciousness, move the victim to open air.  Administer artificial respiration or oxygen as necessary.  Seek medical attention.  If swallowed, give milk or water and induce vomiting.

SECTION VIII-SPECIAL PRECAUTIONS

ALWAYS USE PERSONAL PROTECTIVE EQUIPMENT AND FOLLOW SAFE LABORATORY PRACTICES DURING HANDLING AND STORAGE OF THIS CHEMICAL.  HANDLE ONLY IN WELL VENTILATED AREAS.

THE DATA PROVIDED IS CORRECT TO THE BEST OF OUR KNOWLEDGE.  WE SHALL NOT BE HELD LIABLE FOR ANY DAMAGES RESULTING FROM HANDLING OR CONTACT WITH THIS PRODUCT.