SECTION I-PRODUCT IDENTIFICATION

PRODUCT NAME: DISSOLVED OXYGEN   CAT# 24002

SYNONYMS: N/A

MOLECULAR WEIGHT: N/A

COMPONENTS IN BOTTLES:

BOTTLE A:

Manganese Sulfate solution   	CAS# [7785-87-7]      HAZARD: N/A

BOTTLE B:

Alkaline iodide solution        	CAS# [N/A]        	HAZARD: CORROSIVE

Dropper Bottle  30ml containing 25% sodium iodide solution

BOTTLE C:

Acetic Acid solution  		CAS# [64-19-7]	HAZARD: CORROSIVE/ FLAMMABLE LIQUID

(3) 30 ml Acetic Acid

SECTION II-PHYSICAL DATA

BOILING POINT: N/A	     	  	SP. GRAVITY: N/A

MELTING POINT: N/A             		% VOLATILES: N/A

SOLUBILITY (WATER): yes		APPEARANCE: Bottle A: slight pink, clear liquid

Bottle B: clear, colorless liquid

Bottle C: clear, colorless liquid w/ pungent odor

SECTION III-FIRE AND EXPLOSION HAZARD

FLASH POINT: N/A        		LEL: Not Known    		UEL: Not Known

EXTINGUISHING MEDIA: Dry Chemical, CO2, water

SPECIAL FIRE FIGHTING PROCEDURES: Wear full protective equipment including self-contained breathing apparatus (eye, body, respiratory)

UNUSUAL FIRE HAZARDS: This product contains Acetic Acid which burns readily upon contact with heat sources.

SECTION IV-REACTIVITY DATA

INSTABILITY: Stable at ambient temperature, avoid contact with heat, flame or strong acids or bases.  Store in a cool, dark, dry area away from direct sunlight and possible heat sources.  This product contains Acetic Acid which burns readily in presence of heat sparks or flame.

HAZARDOUS POLYMERIZATION: Unknown

DECOMPOSITION PRODUCTS: Unknown.

INCOMPATIBLES: Bottle B: acids    Bottle C: bases and oxidizers

HAZARD INDEX:

Bottle A:         HEALTH = 0			FLAMMABILITY = 0            	REACTIVITY = 1

Bottle B:         HEALTH = 2			FLAMMABILITY = 0            	REACTIVITY = 1

Bottle C:         HEALTH = 2			FLAMMABILITY = 2            	REACTIVITY = 1

SECTION V-SPILL, LEAK AND DISPOSAL PROCEDURES

SPILL & DISPOSAL: Contain, collect and dispose all of the waste in accordance with applicable local, state and federal laws.

SECTION VI-SPECIAL PROTECTIVE EQUIPMENT

Use in adequately ventilated area,  Utilize protective equipment; chemical splash goggles, rubber gloves and apron. Mechanical hood.

SECTION VII-HEALTH HAZARD DATA

TLV: Not known    MUTAGENICITY: Not known   LD50: Not known

EFFECTS OF OVEREXPOSURE:  Prolonged and repeated contact with this chemical may be harmful and could cause severe burns and/or irritation with contact on the skin, eyes, and mucous membranes.  Always avoid contact with skin, eyes and mucous membranes.  Do not breathe vapors.

FIRST AID: In case of contact with skin, rinse the affected area with large amounts of water for at least 15 minutes.  Wash contaminated clothing before reuse.  If irritation occurs, seek medical advice.  In case of contact with eyes, flush with water for 15 minutes and seek medical attention.  If inhalation of vapors cause loss of consciousness, move the victim to open air.  Administer artificial respiration or oxygen as necessary.  Seek medical attention.

SECTION VIII-SPECIAL PRECAUTIONS

ALWAYS USE PERSONAL PROTECTIVE EQUIPMENT AND FOLLOW SAFE LABORATORY PRACTICES DURING HANDLING AND STORAGE OF THIS CHEMICAL.  HANDLE ONLY IN WELL VENTILATED AREAS.

THE DATA PROVIDED IS CORRECT TO THE BEST OF OUR KNOWLEDGE.  WE SHALL NOT BE HELD LIABLE FOR ANY DAMAGES RESULTING FROM HANDLING OR CONTACT WITH THIS PRODUCT.