SECTION I-PRODUCT IDENTIFICATION

PRODUCT NAME: LEAD TEST KIT      CAT# 82020NP & 82020NPRF

SYNONYMS: N/A

MOLECULAR WEIGHT: N/A

COMPONENTS IN BOTTLES:

REAGENT #1:

Sodium Sulfate	        		CAS# [7757-82-6]           	HAZARD: N/A

Potassium Cyanide			CAS#[151-50-8]		HAZARD: TOXIC

Ammonium Hydroxide		CAS#[1336-21-6]		HAZARD: CORROSIVE

REAGENT #2:

Sodium Hydroxide, dilute    		CAS# [1310-73-2]    		HAZARD: CORROSIVE

DITHIZONE SOLUTION:

Dithizone				CAS#[60-10-6]		HAZARD: N/A

Chloroform		  		CAS# [67-66-3]	           HAZARD: TOXIC

Chloroform		  		CAS# [67-66-3]	           HAZARD: TOXIC

*Please see MSDS for Chloroform for more information

SECTION II-PHYSICAL DATA

BOILING POINT: N/A	     	  	SP. GRAVITY: N/A

MELTING POINT: N/A             		SOLUBILITY (WATER): yes; Chloroform = insoluble

APPEARANCE: Reagent #1: clear, colorless liquid, w/ pungent odor

Reagent #2: clear, colorless liquid

Dithizone Solution: dark purple liquid w/ pungent odor,

Chloroform: clear, colorless liquid w/ pungent odor

SECTION III-FIRE AND EXPLOSION HAZARD

FLASH POINT: N/A        		LEL: Not Known    		UEL: Not Known

EXTINGUISHING MEDIA: Dry Chemical, CO2, water

SPECIAL FIRE FIGHTING PROCEDURES: Wear full protective equipment including self-contained breathing apparatus (eye, body, respiratory)

UNUSUAL FIRE HAZARDS: N/A

SECTION IV-REACTIVITY DATA

INSTABILITY: Stable at ambient temperature, avoid contact with heat, flame or strong acids.  Store in a cool, dark, dry area away from direct sunlight and possible heat sources.

HAZARDOUS POLYMERIZATION: Unknown		DECOMPOSITION PRODUCTS: Unknown.

INCOMPATIBLES: Reagent #1 & 2: acids

HAZARD INDEX:

Reagent A:      HEALTH = 3			FLAMMABILITY = 0            	REACTIVITY = 1

Reagent B:      HEALTH = 3			FLAMMABILITY = 0		REACTIVITY = 1

Dithizone Reagent:	HEALTH = 3		            FLAMMABILITY = 0  		REACTIVITY = 1

Chloroform:	HEALTH = 3			FLAMMABILITY = 0		REACTIVITY = 1

SECTION V-SPILL, LEAK AND DISPOSAL PROCEDURES

SPILL & DISPOSAL: Contain, collect and dispose all of the waste in accordance with applicable local, state and federal laws.

SECTION VI-SPECIAL PROTECTIVE EQUIPMENT

Use in adequately ventilated area (Fume Hood is recommended),  Utilize protective equipment; chemical splash goggles, rubber gloves and apron.

SECTION VII-HEALTH HAZARD DATA

TLV: Not known    MUTAGENICITY: Not known   LD50: Not known

EFFECTS OF OVEREXPOSURE:  Prolonged and repeated contact with these reagents may be harmful and could cause severe burns and/or irritation with contact on the skin, eyes, and mucous membranes.  Always avoid contact with skin, eyes and mucous membranes.  Do not breathe vapors.  Some of these reagents contain Sodium Hydroxide and Ammonium Hydroxide, both of which are very corrosive to the skin, eyes and mucous membranes.  Reagent #1 contains Potassium Cyanide which is very toxic to the skin, eyes and mucous membranes.  Some reagents in this kit contain Chloroform which is a suspect carcinogen and may be irritating upon contact with skin, eyes, mucous membranes.  Handle all reagents with care.

FIRST AID: In case of contact with skin, rinse the affected area with large amounts of water for at least 15 minutes.  Wash contaminated clothing before reuse.  If irritation occurs, seek medical advice.  In case of contact with eyes, flush with water for 15 minutes and seek medical attention.  If inhalation of vapors cause loss of consciousness, move the victim to open air.  Administer artificial respiration or oxygen as necessary.  Seek medical attention.

SECTION VIII-SPECIAL PRECAUTIONS

ALWAYS USE PERSONAL PROTECTIVE EQUIPMENT AND FOLLOW SAFE LABORATORY PRACTICES DURING HANDLING AND STORAGE OF THIS CHEMICAL.  HANDLE ONLY IN WELL VENTILATED AREAS.

THE DATA PROVIDED IS CORRECT TO THE BEST OF OUR KNOWLEDGE.  WE SHALL NOT BE HELD LIABLE FOR ANY DAMAGES RESULTING FROM HANDLING OR CONTACT WITH THIS PRODUCT.