SECTION I-PRODUCT IDENTIFICATION

PRODUCT NAME: NITRATE LOW  CAT# 63005   AND    NITRATE HIGH CAT# 63100

SYNONYMS: N/A

MOLECULAR WEIGHT: N/A

COMPONENTS IN VIAL:

Citric Acid			   			CAS# [5949-29-1]		HAZARD: N/A

Sulfanilimde						CAS# [63-74-1]		HAZARD: N/A

N-(1-napthyl)-ethylenediamine dihydrochloride	CAS# [1465-25-4]		HAZARD: N/A

COMPONENT IN CAPSULE:

Cadmium powder			           		CAS# [7440-43-9]		HAZARD: N/A

SECTION II-PHYSICAL DATA

BOILING POINT: N/A	     	  	SP. GRAVITY: N/A

MELTING POINT: N/A             		% VOLATILES: N/A

SOLUBILITY (WATER): yes		APPEARANCE: Cadmium: metallic grey powder

Vial: white crystalline solid

SECTION III-FIRE AND EXPLOSION HAZARD

Flammability of the Product: May be combustible at high temperature.

Auto-Ignition Temperature: 570°C (1058°F)

EXTINGUISHING MEDIA: Dry Chemical, CO2, water

SPECIAL FIRE FIGHTING PROCEDURES: Wear full protective equipment including self-contained breathing apparatus (eye, body, respiratory)

UNUSUAL FIRE HAZARDS: Not Known.

SECTION IV-REACTIVITY DATA

INSTABILITY: Stable at ambient temperature, avoid heat, flames and other heat sources.  Store in a cool, dark, dry area away from direct sunlight.

HAZARDOUS POLYMERIZATION: Will not occur

DECOMPOSITION PRODUCTS: Unknown.

INCOMPATIBLES: Capsule: N/A                 Vial: strong bases

HAZARD INDEX:

Contents in vial:	 HEALTH = 2		FLAMMABILITY = 1            REACTIVITY = 1

Cadmium Vial	:    	HEALTH = 3		FLAMMABILITY = 1	 REACTIVITY = 0

SECTION V-SPILL, LEAK AND DISPOSAL PROCEDURES

SPILL & DISPOSAL: Contain, collect and dispose all of the waste in accordance with applicable local, state and federal laws.

SECTION VI-SPECIAL PROTECTIVE EQUIPMENT

Use in adequately ventilated area,  Utilize protective equipment; chemical splash goggles, rubber gloves and apron. Mechanical hood.

SECTION VII-HEALTH HAZARD DATA

TLV: Not known    MUTAGENICITY: Not known   LD50: Not known

EFFECTS OF OVEREXPOSURE (Vial):  Prolonged and repeated contact with this chemical may be harmful and could cause severe burns and/or irritation with contact on the skin, eyes, and mucous membranes.  Do not breathe dusts.

EFFECTS OF CADMIUM EXPOSURE

Hazardous in case of ingestion, of inhalation. Slightly hazardous in case of skin contact (irritant, sensitizer), of

eye contact (irritant). Severe over-exposure can result in death.

FIRST AID: In case of contact with skin, rinse the affected area with large amounts of water for at least 15 minutes.  Wash contaminated clothing before reuse.  If irritation occurs, seek medical advice.  In case of contact with eyes, flush with water for 15 minutes and seek medical attention.  If inhalation of dusts/vapors cause loss of consciousness, move the victim to open air.  Administer artificial respiration or oxygen as necessary.  Seek medical attention.

OTHER REGULATORY VIII

TSCA 8(b) inventory: Cadmium

SARA 313 toxic chemical notification and release reporting: Cadmium

CERCLA: Hazardous substances.: Cadmium

SECTION IV-SPECIAL PRECAUTIONS

ALWAYS USE PERSONAL PROTECTIVE EQUIPMENT AND FOLLOW SAFE LABORATORY PRACTICES DURING HANDLING AND STORAGE OF THIS CHEMICAL.  HANDLE ONLY IN WELL VENTILATED AREAS.

THE DATA PROVIDED IS CORRECT TO THE BEST OF OUR KNOWLEDGE.  WE SHALL NOT BE HELD LIABLE FOR ANY DAMAGES RESULTING FROM HANDLING OR CONTACT WITH THIS PRODUCT.