## SECTION I - PRODUCT IDENTIFICATION

PRODUCT NAME: ZINCON (VIALS)

CHEMICAL NAME & SYNONYMS (VIALS): Zincon; 2-carboxy-2’-hydroxy-5’-sulfoformazylbenzene

FORMULA: C20H15N406SNa

CAS# 			SARA TITLE 313		DOT CLASS		REPORTABLE QUANTITY

[62625-22-3]		NO				NR			N/A

### SECTION -PHYSICAL DATA

### Melting point 250 C; Slightly soluble in water; readily soluble in aqueous alkali.

APPEARANCE & ODOR

Deep violet powder. Odorless

### SECTION - FIRE HAZARD

Not flammable. Will be consumed in a general fire. Avoid inhalation of fumes. Firefighters wear SCBA.

Extinguisher: Water, CO2 or dry chemical

SECTION - REACTIVITY DATA

No hazardous self-reaction. Incompatible with strong oxidizing agents.

### SECTION - SPILL, LEAK AND DISPOSAL PROCEDURES

SPILL & DISPOSAL: Contain, collect and dispose all of the waste in accordance with applicable local, state and federal laws.

### SECTION - HEALTH HAZARD DATA

Will stain skin on contact. Eye irritant. May be irritating to skin and mucous membranes. The toxicological properties have not been investigated. LD/TD no data found. OSHA PEL/ACGIH TLV not established. No evidence of carcinogenicity.

FIRST AID

Wash skin or eyes thoroughly with water after contact. If eye irritation persists, get medical attention. If ingested, give water or milk to drink and get medical attention.

### SECTION  -SPECIAL PRECAUTIONS

ALWAYS USE PERSONAL PROTECTIVE EQUIPMENT AND FOLLOW SAFE LABORATORY PRACTICES DURING HANDLING AND STORAGE OF THIS CHEMICAL.  WEAR EYE PROTECTION AND RUBBER GLOVES WHILE HANDLING. USE IN WELL VENTILATED AREA OR USE SUITABLE RESPIRATOR IF DUST IS RAISED.

THE DATA PROVIDED IS CORRECT TO THE BEST OF OUR KNOWLEDGE.  WE SHALL NOT BE HELD LIABLE FOR ANY DAMAGES RESULTING FROM HANDLING OR CONTACT WITH THIS PRODUCT.

CHEMICAL NAME & SYNONYMS – CYCLOHEXANONE (dropper bottle)

Cyclohexanone

FORMULA: C6H10O

CAS# 			SARA TITLE 313		DOT CLASS		REPORTABLE QUANTITY

108-94-1		No				3 / III			2270 kg

PHYSICAL DATA

Melting point -47 C; Boiling point 155 C; Specific Gravity 0.95; Vapor Density 3.4. Vapor pressure 2 mm at 20 C.

APPEARANCE & ODOR

Clear, colorless liquid. Characteristic odor.

REACTIVITY & CONDITIONS TO AVOID

Stable. Combustible. No hazardous self-reactivity. Incompatible with strong oxidizers.

FIRE HAZARDS

Combustible. Will burn readily if ignited. NFPA #2-2-1

EXTINGUISHER						FLASHPOINT	LEL	UEL

CO2, Water, Dry chemical					44			1.1%	9.4%

HEALTH HAZARDS

Toxic by inhalation, ingestion or skin contact. Irritating to skin, eyes and mucous membranes – can cause serious eye irritation. Narcotic effects. LD50 (oral –rat) 1535 mg/kg.; OSHA PEL TWA: 50 ppm; ACGIH TLV: TWA 25ppm, STEL 100 ppm. Possible mutagenic, reproductive effects. No evidence of carcinogenicity.

SPECIAL PRECAUTIONS

Wear goggles and rubber gloves while handling. Use only with suitable ventilation. Avoid ignition sources.

FIRST AID

Wash skin and eyes thoroughly with water after contact. If irritation persists, see a physician. If swallowed, give water or milk an induce vomiting. Inhalation – remove to fresh air, if breathing is difficult, give oxygen; if breathing stops, give respiration. Get immediate medical assistance.

SPILLS & LEAKS

Remove ignition sources. Ventilate area. Contain liquid and absorb in suitable absorbent. Disposal to approved incinerator. Do not dispose to sewer.